MONASH HEALTHCARE INNOVATION SUMMER SCHOLARSHIPS (HISS)

PROJECT NAME: Nemo

DELIVERABLES NUMBER: 2

PROJECT LEAD:  David Levitz & Vishnu Sivakumar

Existing Solutions

| Overview High-level description Who uses it Skill required by the user | Clinical Solution Profile Describe clinical rationale / indication Mechanism of action Clinical evidence for safety and efficacy Patient segments | Economic Solution Profile Cost (including both direct and ancillary costs) Who / what entity pays for it | Utilisation Solution profile Describe exactly how it’s used in clinical practice (step-by-step), by whom, and where | Emerging Solution profile If it’s a relatively novel technology, look at how it’s going to impact the solution landscape | Gaps Compare the solution to each dot-point of the acceptance criteria, describe whether it doesn’t address it. |
| --- | --- | --- | --- | --- | --- |
| Trial & Error Using Major Anatomical Landmarks - Used by neurologists or clinicians without any screening equipment - Positions of nerve locations are marked based on common human anatomy - When conducting tests on patients with variant body habitus/anatomy, clinicians require a high amount of skill to be able to approximate the position of the nerve to stimulate it at the right place - Manipulation of patient position | - Only 7-41% of the population have minor anomalies, hence the majority of patients seen on a day to day basis can be assessed based on major anatomical landmarks - Non-invasive technique. Therefore, the chance of infection is very low. - No extra costs - No extra equipment | - No extra equipment required (resulting in no extra cost) - Takes longer for clinicians to perform the study in the case of an uncommon anatomy (resulting in extra cost for the government/patient) | 1) Patients are referred to neurologists depending on the type of problem they are experiencing 2) Neurologists attach the recording electrodes based on major anatomical landmarks 3) By using major anatomical landmarks, clinicians mark positions that are known to have nerves running under them. 4) Using nerve conduction electrodes, neurologists send a small electric pulse at the marked site. | NOT APPLICABLE This solution has been widely practised for a long time. | - Time inefficient Clinicians may have to stimulate several times at multiple places to find the super-maximal amplitude of response. Which can be time-consuming for both patients and clinicians. If the results are ambiguous, clinicians may refer the patients to radiologists to conduct various other tests to confirm diagnosis/prognosis. - Inaccurate & not reliable - No guarantee that the super-maximal point used is exactly over the nerve fibre as the doctor has very few chances to trial and error before they hit the patient's 'physical limit'. - A different result may be obtained each time the test is performed. |
| Screening Techniques (Ultrasound) - Used by a neurologist also trained in using ultrasound machinery or by a sonographer - Scans over the area of interest are taken and the position of the nerve is marked. The nerve conduction study is then conducted at the marked position. - Specialised training is required to operate an ultrasound, although the process is relatively simple. | - Screening techniques, especially ultrasound have been used for a very long time and have been proven to have no significant side effects. - Techniques such as ultrasound have been proven to also detect nerves. - By using nerve conduction testing in conjunction with screening techniques, clinicians can be relatively more time efficient. Furthermore, they can also be more confident in their diagnosis. - In most cases, no injuries or major side-effects have been reported. Although, some techniques such as x-ray do pose a threat as they use ionising radiation which may cause tissue damage or increase a patient's chance of developing cancer. | - The cost of performing an ultrasound is $250 - $350 AUD per session - Depending on the reason for screening, medicare covers a percentage of the cost with the patient having to pay for the rest. | - A conductive gel is applied over the site of interest by the neurologist or the sonographer. - The clinician then scans the area using a transducer and marks the position of the nerve. - Nerve conduction testing is then performed over the marked spot. - Usually performed at a radiology clinic, although if the neurology clinic has access to an ultrasound, then the neurologist may perform the ultrasound if they have been trained. | - Storage must be organised within the neurology clinic | - Do not have access to an ultrasound - Expensive equipment - Time inefficient (ultrasound must be set up before use). Although, relatively better than trial and error method. |
| Pre-Configured NSE Array - Used by neurologist who perform nerve conduction studies - Little to no skill required as instructions and positions are labelled on the unit | - Relatively simple procedure - Most hygienic solution as it is one time use only. - Inexpensive equipment - Potential fire hazard on the wrist. If the circuit malfunctions, the paperlike sticker may catch fire. | NOT APPLICABLE - Product has still not been released for the general public use. - Value of the product has not been published in the article. | - Can be performed by a regular neurologist in a neurology clinic - sticker is placed over the patient's body (depending on where the test is being conducted) - Using the keyed connector, the clinician connects the embedded recording electrodes to a measuring/monitoring device. - Apparatus is removed and disposed of. | - Clinicians have to purchase multiple types of arrays as they are nerve specific. - Storage must also be organised within the clinic. | - Can still be inaccurate as a person with variant anatomy may have nerves positioned outside the stimulation zone. - Can be time inefficient as a clinician must align each of the markings over the patient's body |
| Transcutaneous electrical nerve locator Conductive electrode is attached to the clinician's finger and pulses are applied as the clinician searches for the nerve. Target nerve is stimulated to indicate where it is located. | -Designed for peripheral nerve blocks - Traditionally anatomically guided as in the nerve conduction studied | Not applicable: Not commercially available. Looks cheap. | Not applicable - would be used by clinician. Minimal training | Limited to only motor nerve |  |
| NERVE LOCATOR/ STIMULATOR Sterile, disposable, portable and self powered, self testing instrument for locating and stimulate ing motor nerves Easy to use Likely most relevant to surgeons as it is invasive | Indication: Where the identity of the tissue is unknown. Stimulator is used to identify the nerve. This has potential in nerve release surgeries (particularly if there has been a history of previous surgery which has disrupted the anatomy. Furthermore, it can be used to identify nerves that must not be cut. MOA: Insertion of one end of the electrode in the subcutaneous tissue to stimulate the nerve and insertion of other end of electrode to ground Concept without clinical application | NOT APPLICABLE - Product has still not been released for the general public use. - Value of the product has not been published | Unlikely to be used as invasive. Probe could be inserted in subcutaneous tissue and motor nerve stimulated when found Would have to be used by neurologist but invasive nature may require anaesthetics | Limited in the Neurology space. Even if more accurate the pain is likely not acceptable | Time efficient + accurate Increased discomfort to patient Likely to be cost efficient as less equipment is required |
| DIGITAL DISTANCE MEASURER FOR NERVE CONDUCTION STUDIES Electronically measure distance and automatically transfer results | Currently nerve conduction studies involve using a tape measure to measure distances and manually typing results into a computer This is highly susceptible to error and is time consuming The device looks to Electronically measure distance and automatically transfer results Contact wheel can measure distances without disruption from conduction gel or disruption of electrical stimuli | NOT APPLICABLE - Product has still not been released for the general public use. - Value of the product has not been published In private = likely clinicians In public = Likely hospital | Clinician uses it as normal. It is used to measure distance accurately. Allows to deviate with the normal folds of skin | May increase the accuracy and speed of nerve conduction tests Reduces potential for human error by automating the process | Much more time efficient - usually 8 recording of upper limb and 7 recordings of lower limb Increased accuracy compared to manually using a tape measure and reducing chance of human error |
| APPARATUS AND METHOD FOR THE AUTOMATED MEASUREMENT OF SURAL NERVE CONDUCTION VELOCITY AND AMPLITUDE Commercially: DPN check | Aids in early detection and monitoring of peripheral neuropathy Measures sural nerve conduction and amplitude In a study of 72 patients with diabetes and confirmed diabetic neuropathy, the correlation of NC‐stat* sural nerve conduction with laboratory methods ranged from 0.87 to 0.95. 2,3 The accuracy, reliability and utility of DPNCheck has been established with over 25 studies published in peer-reviewed journals. The following table addresses sensitivity and specificity. (see ppt) | Cost depends on the private or public system. Private = the clinic Public = the hospital Costs about 5,500 dollars | 1. Position patient + prepare skin 2. Place biosensor and apply conductive gel 3. Position device and start test 4. Use of plurality of electrodes to detect the sural nerve 5. Results in less than 15 seconds 6. Electronically digitalised, processed and stored | In a study of 195 patients with Type I diabetes,1 100% of patients with a positive monofilament test also had abnormal sural nerve conduction†. However, 60% of patients with a negative monofilament test actually had neuropathy as defined by abnormal sural nerve conduction. The variation should be less than 5% for conduction velocity and 25% for amplitude.6 Look at data for patients with large habitus and soft tissue | Efficient and quick Accurate Cost effective |

Solution Alignment

| Name | Cost neutral/reducing | Time neutral/reducing | Doesn’t require a lot of specialised training | Reliability/Accuracy of results | Patient Comfort |
| --- | --- | --- | --- | --- | --- |
| Trial & Error |  |  |  |  |  |
| Screening Techniques |  |  |  |  |  |
| Pre-Configured Array |  |  |  |  |  |
| Transcutaneous Nerve Locator |  |  |  |  |  |
| Nerve Locator/Stimulator |  |  |  |  |  |
| Distance Measurer |  |  |  |  |  |
| Automated Measurement |  |  |  |  |  |

State of The Art Summary

Problem:

Nerves located in positions that are unorthodox.

Population:

Patients who have variant body habitus/anatomy or are suffering from conditions such as edema or obesity.

Pain Point:

Stimulate, multiple times, around the approximate location of the nerve.

Currently in clinics, nerve conduction studies (NCS) are used as a standalone technique. Patients experiencing issues with body movements are referred to neurologists by their general practitioner. Upon which, the neurologist performs the nerve conduction testing on the set patient. However, for the small fraction of patients who have variant body habitus/anatomy or are suffering from conditions such as edema or obesity have nerves located in positions that are unorthodox. Therefore, the neurologist performing the NCS will have to stimulate, multiple times, around the approximate location of the nerve. Due to repeated artificial electric pulses, this process can be an unpleasant experience for patients.

Nonetheless, the clinician will then use this method to find the super-maximal point of amplitude and mark it. The neurologist will then perform the NCS at the marked site. Although, due to ambiguities in the results, the clinician may refer the patient to a radiologist and recommend other tests to confirm diagnosis/prognosis. Which can be deemed as time inefficient as both, the patient’s and clinician’s time is wasted on a procedure which is only 50% accurate, leading to them spending time on other tests which could have otherwise been avoided.

As a way to improve the procedure of NCS and its accuracy of results, it can potentially be combined with a screening technique to localise nerves. By using a screening technique, such as an ultrasound, neurologists will easily be able to locate nerves and stimulate it accurately. Consequently, making the procedure more comfortable for patients.

Technologies & Services Relevant to Project NEMO:

- Ultrasound

- NCS